[doc sps="1.8" acron="rconst" jtitle="Revista de la construcción" stitle="Revista de la Construcción" issn="0718-915X" eissn="0718-915X" pubname="Escuela de Construcción Civil, Pontificia Universidad Católica de Chile" license="http://creativecommons.org/licenses/by-nc/4.0" volid="21" issueno="2" dateiso="20220900" season="Sep./Dec" order="01" fpage="193" lpage="203" pagcount="11" doctopic="oa" language="en"]

DOI:[doi]10.7764/RDLC.21.2.193[/doi]

[toctitle]RESEARCH ARTICLE[/toctitle]

[doctitle]Recent hydraulic bridge failures in China: review and discussion[/doctitle]

[author role="nd"][fname]Sifeng[/fname] [surname]Qin[/surname][xref ref-type="aff" rid="aff1"]1[/xref] [authorid authidtp="orcid"]https://orcid.org/0000-0002-4470-0290[/authorid][/author]

[author role="nd"][fname]Huili[/fname] [surname]Wang[/surname][xref ref-type="aff" rid="aff2"]2[/xref], [xref ref-type="aff" rid="aff3"]3[/xref]  [authorid authidtp="orcid"]https://orcid.org/0000-0002-0675-2524[/authorid][/author]

[author role="nd"][fname]Zhikang[/fname] [surname]Luo[/surname][xref ref-type="aff" rid="aff1"]2[/xref][authorid authidtp="orcid"]https://orcid.org/0000-0001-5182-6134[/authorid][/author]

[normaff id="aff1" ncountry="Not normalized" norgname="Not normalized" icountry="CN"][label]1[/label]College of Civil Engineering and Architecture, [orgname]Dalian University[/orgname], [city]Dalian [/city]([country]China[/country]), [email]dr.qinsifeng@hotmail.com[/email][/normaff]

[normaff id="aff2" ncountry="China" norgname="Dalian University of Technology" icountry="CN"][label]2[/label]National & Local Joint Engineering Laboratory of Bridge and Tunnel Technology, [orgname]Dalian University of Technology[/orgname], [city]Dalian [/city]([country]China[/country]), [email]wanghuili@dlut.edu.cn[/email]; [email]luozhikang88@qq.com[/email][/normaff]

[normaff id="aff3" ncountry="China" norgname="Dalian University of Technology" icountry="CN"][label]3[/label]State Key Laboratory of Structural Analysis for Industrial Equipment, [orgname]Dalian University of Technology[/orgname], [city]Dalian [/city]([country]China[/country])[/normaff]

[corresp id="c1"]Correspondence: [email]wanghuili@dlut.edu.cn[/email][/corresp]

[xmlabstr language="en"]Abstract: This paper studies 123 hydraulic bridge failures in China from 1998 to 2018. The geographic distribution, age distribution, bridge type distribution, time distribution and the hydraulic bridge failure causes are analyzed. Six typical failure cases of hydraulic bridges are analyzed in detail. 100 hydraulic bridge failures occurred in southern China so hydraulic bridge failures are much more common in this area. The average service life of these bridges is 28.9 years. The beam bridge accounts for the largest proportion of hydraulic bridge failure because 71 (57.7%) beam bridges were destroyed by flood. Hydraulic bridge failure mainly occurred between 2009 and 2014, which was related to the impact of the Wenchuan earthquake. The incidents major causes include unexpected flood, earthquake, old bridge, over sand exploitation, low awareness of hydraulic bridge damage, extreme morphology of the river, etc. There are both natural factors and man-made factors. These results show that three aspects should be paid attention to: (1) unexpected flood; (2) appropriate bridge type; and (3) extreme morphology of the river.[/xmlabstr]

[kwdgrp language="en"][sectitle]Keywords:[/sectitle] [kwd]Bridge[/kwd], [kwd]hydraulic failure[/kwd], [kwd]cause[/kwd], [kwd]China[/kwd][/kwdgrp].

[hist]Received: [received dateiso="20201104"]04.11.2020[/received]; Accepted: [accepted dateiso="20220324"]24.03.2022[/accepted]; Published: 30.08.2022[/hist]

- [xmlbody][sec sec-type="intro"][sectitle]Introduction[/sectitle]

[p]There are many bridge failures in the world. Bridge failure is the most concerned problem of the engineers (Wesley [xref  ref-type="bibr" rid="r2"]Cook, Bar, & Halling, 2015[/xref]; [xref  ref-type="bibr" rid="r7"]Harik, Shaaban, Gesund, Valli, & Wang, 1990[/xref]; [xref  ref-type="bibr" rid="r18"]Rodrigo, Olària, Fernández-Ordoñez, & Gómez, 2015[/xref]). The consequences of bridge failure can range from unexpected maintenance needs, to the loss of life and economic prosperity. Failure types include hydraulic, collision, overload, deterioration, fire, construction, fatigue, and miscellaneous ([xref  ref-type="bibr" rid="r4"]Duntemann & Subrizi, 2000[/xref]; [xref  ref-type="bibr" rid="r17"]Moroni, Sarrazin, Venegas, & Villarroel, 2015[/xref]; [xref  ref-type="bibr" rid="r20"]Wardhana & Hadipriono, 2003[/xref]). [/p]

[p]The damage caused by foundation scour and settlement from soil erosion is a great threat to bridge structure. Scour is the main cause of bridge failure ([xref  ref-type="bibr" rid="r4"]Duntemann & Subrizi, 2000[/xref]; [xref  ref-type="bibr" rid="r7"]Harik et al., 1990[/xref]). Wadhana found that flooding or scouring was the main cause of bridge failure in the United States ([xref  ref-type="bibr" rid="r20"]Wardhana & Hadipriono, 2003[/xref]). Cook estimated an annual hydraulic bridge failure frequency of approximately 1/8500 (W. Cook, Barr, & Halling, 2014). Montalvo reported that in the United States, 55% of collapsed bridges were caused by hydraulic collapse (Montalvo, Cook, & Keeney, 2020). Flint indicated that bridge failures may increase due to more frequent or intense flooding ([xref  ref-type="bibr" rid="r13"]Madeleine M. Flint, Oliver  Fringe, Sarah L. Billington, David Freyberg, & Diffenbaugh, 2017[/xref]). Lin selected 45 bridges as a case study to evaluate scour performance using the developed integrated analysis technique (Lin, Bennett, Han, & Parsons, 2012). Hung examined the influence of scour on the behavior of bridge piers subjected to flood-induced loading(Hung & Yau, 2014). Hager investigated the end scour depth due to a single peaked flood wave in an essentially plane rectangular sediment bed containing a single pier for clear-water conditions(Hager & Unger, 2010).[/p]

[p]Foundation settlement is caused by erosion and weak soil conditions. When the pier tilts, there is potential for the bridge to collapse without warning. After the construction of piers and abutments, the cross-section area of flood decreases and the velocity of flow increases, which causes local scour around the piers and makes the foundation empty. In recent years, global climate change has led to the gradual concentration of rainwater and the frequent occurrence of floods. Once the foundation without sufficient depth is submerged, it is easy to collapse. Today, climate change and associated extreme weather events are causing more flood-related damage to bridges([xref  ref-type="bibr" rid="r1"]Choudhury & Hasnat, 2015[/xref]; [xref  ref-type="bibr" rid="r20"]Wardhana & Hadipriono, 2003[/xref]). Most of the damage are not just caused by water. During the flood period, sediment and other tiny floating objects are easy to deposit in the river channel, which affects the flood discharge and makes the bridge easy to be washed away by water. Large floating objects, such as trees, will directly impact the bridge foundation and damage the pier and river channel.[/p]

[p]By the end of 2017, more than 830000 bridges had been built in China ([xref  ref-type="bibr" rid="r25"]Zhou & Zhang, 2019[/xref]). Bridge failures often occur in China ([xref  ref-type="bibr" rid="r12"]Hong, Chiew, Lu, Lai, & Lin, 2012[/xref]; [xref  ref-type="bibr" rid="r10"]Ji & Fu, 2010[/xref]; D. [xref  ref-type="bibr" rid="r22"]Xu, 2003[/xref]; H. T. Xu, Guo, Pu, & Yuan, 2007; [xref  ref-type="bibr" rid="r26"]Zhuang, Xiao, Jia, & Sun, 2020[/xref]). In this paper, the diseases of hydraulic bridge in China from 1998 to 2018 are analyzed retrospectively. The investigation constitutes the types and causes of hydraulic bridge failure in China. [/p]

- [/sec][sec sec-type="cases"][sectitle]Case study of hydraulic bridge failure[/sectitle]

[p]This paper discusses the hydraulic bridge failure in China from 1998 to 2018.[/p]

  - [p]Taiwan Gaoping Bridge[/p]

[p]Taiwan Gaoping Bridge was a continuous beam bridge and about 1990 meters long. On August 27, 2000, severe tropical storm Bilis destroyed some piers of Gaoping Bridge. The bridge deck collapsed about 100 meters, a total of 17 vehicles collided on the road, and 22 people were seriously injured, as shown in [xref  ref-type="fig" rid="f1"]Figure 1[/xref] (Sina, 2000).   [/p]

[figgrp id="f1"][graphic href="?art01"][/graphic]

[label]Figure 1. [/label] [caption]Collapse of Gaoping Bridge.[/caption][/figgrp]

  - [p]Sichuan Second Qujiang Bridge[/p]

[p]The Second Qujiang Bridge was an arch bridge, located in Dazhou City, Sichuan Province. It was 573 meters long and 15.5 meters wide. The construction began in 2008 and had expected to be completed in 2011. But in July 2010, after two-thirds of the total construction were completed, the project was destroyed by floods, as shown in Figure 2 (Xinhuashe, 2010). The bridge was under construction, so it was an unintegrated structural system. The instability of the bridge and the erosion caused by the over [uri href="javascript:;"]expectation[/uri] flood leaded to the destruction of the bridge. Pearson-Ⅲ theoretic frequency curves are widely used to calculate flood rate in China. The design flood rate prediction is based on historical measurements. More historical measurements can improve prediction precision.[/p]

[figgrp id="f2"][graphic href="?art01"][/graphic]

[label]Figure 2. [/label] [caption] Collapse of Second Qujiang Bridge.[/caption][/figgrp]

  - [p]Tangying Bridge[/p]

[p]On July 24, 2010, Tangying Bridge in Luoyang City, Henan Province collapsed due to rainstorm, resulting in 53 deaths and 13 missing, as shown in Figure 3 (Zhaoting, 2010) .[/p]

[p]Tangying Bridge was a continuous beam bridge with 233.7 meters long and 8 meters wide. The bridge, built in the 1980s, was a 5-span continuous arch bridge with a net span of 40 meters. It was made of stone and mortar without any reinforcement. This type of bridge structure is sensitive to foundation settlement. Any skewback foundation settlement will produce enormous secondary internal force. The flood scoured a skewback of the bridge and caused foundation settlement. Then the bridge collapsed under the action of a huge secondary internal force.[/p]

[figgrp id="f3"][graphic href="?art01"][/graphic]

[label]Figure 3. [/label] [caption] Collapse of Tangying Bridge.[/caption][/figgrp]

  - [p]Shiting Bridge[/p]

[p]On August 19, 2010, Shiting Bridge of Baocheng railway was destroyed by flood, resulting in two sections of K165 train falling into the river, as shown in Figure 4 ([xref  ref-type="bibr" rid="r14"]Mengqi, 2010[/xref]). Fortunately, there were no casualties.[/p]

[figgrp id="f4"][graphic href="?art01"][/graphic]

[label]Figure 4. [/label] [caption] Failure of Shiting River Bridge.[/caption][/figgrp]

[p]This bridge was constructed in the 1950s. It was a continuous beam with a net span of 16m. The piers were made of stone and the piles were made of wood. The pile cover was only 5 meters deep. Due to the weak discharge capacity of small span bridges, shallow foundations are particularly prone to erosion. Then two piers of the bridge were inclined due to the flood. When the train passed the bridge, two piers of the bridge collapsed.  [/p]

  - [p]Mianzu Bridge[/p]

[p]Mianzu Bridge, a continuous concrete beam, was located in Mianzu City across the Shiting River. It was 366.5 meters long and 10.75 meters wide. On August 18, 2012, two piles disappeared due to flood scour, as shown in Figure 5 ([xref  ref-type="bibr" rid="r5"]Guang, 2012[/xref]). The dashed lines represent the disappeared piles in Figure 5.  [/p]

[figgrp id="f5"][graphic href="?art01"][/graphic]

[label]Figure 5. [/label] [caption] Failure of Mianzu Bridge.[/caption][/figgrp]

[p]The 2008 Wenchuan earthquake caused a large number of gravel and finger stones to fall into the Shiting River, which resulted in narrow water way increasing the flow velocity downstream of blocked part of the river. This enhanced the scouring effect. In addition, the excessive exploitation of river sediment aggravated the decline of river stage. Therefore, the piles were washed away by the flood.[/p]

  - [p]Huaiyuan Bridge[/p]

[p]Huaiyuan Bridge was a multi-span simply supported beam bridge with a total length of 135 meters. On August 19, 2010, the bridge collapsed due to flood, as shown in Figure 6 ([xref  ref-type="bibr" rid="r15"]Mingkang, 2010[/xref]). More than 60 meters of the bridge fell into the river. Two people died.  [/p]

[figgrp id="f6"][graphic href="?art01"][/graphic]

[label]Figure 6. [/label] [caption] Collapse of Chongzhou Huaiyuan Bridge.[/caption][/figgrp]

[p]The bridge was built in the 1960s and was identified as an unsafe bridge with limited bearing capacity in 2008. After the 2008 Wenchuan earthquake, the river became narrow due to the deposition of finger stones. As a result, the flow velocity was accelerated and the scour was intensified. This undermined the bridge foundation. Then the bridge was destroyed by the flood.[/p]

  - [p]Other cases of hydraulic bridge failure[/p]

[p]Some typical cases of hydraulic bridge failure are listed in [xref  ref-type="table" rid="t1"]Table 1[/xref].[/p]

[tabwrap id="t1"][label]Table 1. [/label] [caption]Cases of hydraulic bridge failure.[/caption]

[graphic href="?art01"][/graphic][/tabwrap]

[figgrp id="ch1"][graphic href="?art01"][/graphic][/figgrp]

- [/sec][sec sec-type="discussion"][sectitle]Discussion[/sectitle]

  - [p]Geographic distribution of hydraulic bridge failure[/p]

[p]From 1998 to 2018, 123 bridge disasters by the flood have occurred in China, as shown in Table 2.[/p]

[tabwrap id="t2"][label]Table 2. [/label] [caption] Distributionof bridge disasters by flood in China.[/caption]

[graphic href="?art01"][/graphic][/tabwrap]

[p]The data is plotted on the map of China, as shown in Figure 7.[/p]

[figgrp id="f7"][graphic href="?art01"][/graphic]

[label]Figure 7. [/label] [caption]Distribution of damaged bridges across the country.[/caption][/figgrp]

[p]Sichuan Province has the highest number of hydraulic bridge failure, which is related to the impact of the Wenchuan earthquake. The 2008 Wenchuan earthquake led to a large number of sand and finger stones falling into the river, which resulted in narrow water way increasing the flow velocity downstream of blocked part of the river. Furthermore, the breached part of blocked water way caused rapid increase in velocity of sediment laden flow which had sufficient drag force and bed shear stress to further erode the bed than normal scour. ([xref  ref-type="bibr" rid="r11"]Ko, Chiou, Tsai, & Chen, 2014[/xref]). In addition, some bridges have been damaged in varying degrees in the earthquake.[/p]

[p]There were 100 hydraulic bridge failures in southern China and 23 hydraulic bridge failures in northern China, resulting in 67 deaths and 138 injuries. The results are listed in Table 3.[/p]

[tabwrap id="t3"][label]Table 3. [/label] [caption] Hydraulic bridge failures in southern China and northern China.[/caption]

[graphic href="?art01"][/graphic][/tabwrap]

[p]The average annual failure probability of hydraulic bridge in southern China is higher than that in northern China. This is due to the frequent heavy rain in southern China and the high probability of flood occurrence. However, the casualty rate of hydraulic bridge failure in southern China is lower than that in northern China. This is because the awareness of hydraulic bridge damage in northern China is relatively weak.[/p]

  - [p]Age distribution analysis[/p]

[p]The service life distribution of 123 bridges investigated is shown in Figure 8. The average service life of these bridges is 28.9 years. Guigou Bridge in Henan Province survived only one day because of the flood. However, the life of Dongmen Bridge in Guangdong Province is 82 years, and that of Qingdao Trestle Bridge in Shandong Province is 121 years. The life of most bridges is less than 35 years and cannot reach the service life.[/p]

[figgrp id="f8"][graphic href="?art01"][/graphic]

[label]Figure 8. [/label] [caption] Age distribution of failed bridges.[/caption][/figgrp]

  - [p]Time distribution of the hydraulic bridge failure[/p]

[p]The time of the hydraulic bridge failure is summarized, as shown in Figure 9. It mainly occurred between 2009 and 2014. During this period, the most hydraulic bridge failures happened in Sichuan Province, which was related to the impact of the Wenchuan earthquake. Some bridges were damaged in the Wenchuan earthquake and did not find or repair in time.[/p]

[figgrp id="f9"][graphic href="?art01"][/graphic]

[label]Figure 9. [/label] [caption]Time distribution of failed bridges.[/caption][/figgrp]

  - [p]Bridge type distribution of the hydraulic bridge failure[/p]

[p]There are 71 beam bridges, 20 arch bridges, 2 suspension bridges and 1 cable-stayed bridge among the investigated bridges. The type of the other 29 bridges is unknown, as shown in Figure 10.[/p]

[figgrp id="f10"][graphic href="?art01"][/graphic]

[label]Figure 10. [/label] [caption] Bridge type distribution of failed bridges.[/caption][/figgrp]

[p]From 2000 to 2018, beam bridges accounted for the biggest share of hydraulic bridge failure. Suspension bridge and cable-stayed bridge take up a small proportion. Beam bridges have more piers and are prone to water scouring, leading to collapse. Arch bridges are generally built in mountainous areas. They are prone to collapse during floods or mudslides. In suspension bridges and cable-stayed bridges, cable is the main load-bearing structure, they are larger than the beam bridge span, suitable for building in the water depth, not easy to be destroyed by water.[/p]

- [/sec][sec][sectitle]Summary and recommendations[/sectitle]

[p]This paper investigates 123 hydraulic bridge failures. The geographic distribution, age distribution, bridge type distribution and time distribution are analyzed. It is found that there are more hydraulic bridge failures in southern China. Sichuan Province has the highest number of hydraulic bridge failure. Most bridges have a life span of less than 35 years. The average service life of these bridges is 28.9 years. Hydraulic bridge failures mainly occurred between 2009 and 2014. The beam bridge accounts for the biggest share of hydraulic bridge failures. There are both natural factors and man-made factors, such as unexpected flood, earthquake, old bridge, over sand exploitation, low awareness of hydraulic bridge damage, extreme morphology of the river, etc. Finally, the findings of this study are as followings:[/p]

[list listtype="order"][li]1. Most of the collapses occurred on old bridges. They cannot be assumed to meet modern design standards. Therefore, the bridge design engineer must adequately consider the expected flood;[/li]

[li]2. The bridge type with fewer piers should be selected, such as cable-stayed bridge and suspension bridge, so as to avoid hydraulic damage. The estimated pier scour depth is not sufficient and doesn’t consider all factors responsible for large scour;[/li]

[li]3. Before construction of any bridge on river, possible changes in extreme morphology of the river should be examined, considering all the natural and anthropogenic factors.[/li][/list][/sec][/xmlbody]

[ack][sectitle]Author contributions: [/sectitle] Sifeng Qin: investigation, methodology, project administration, supervision. Huili Wang: data curation, conceptualization, methodology. Zhikang Luo: writing - review & editing.[/ack]

[ack][sectitle]Funding: [/sectitle] This work is supported by the Liaoning Provincial Doctoral Scientific Foundation Projects (20170520138) and the Liaoning Provincial Natural Science Foundation Guidance Projects (2019-ZD-0006). [/ack]

[ack][sectitle]Acknowledgments: [/sectitle] The authors contributed jointly to all phases of the article. [/ack]

[ack][sectitle]Conflicts of interest: [/sectitle] We declare that there is no conflict of interest for the article [/ack].

[refs][sectitle]References[/sectitle]

[ref id="r1" reftype=“confproc"][authors role="nd"][pauthor][surname]Choudhury[/surname], [fname]J. R.[/fname][/pauthor], & [pauthor][surname]Hasnat[/surname], [fname]A[/fname][/pauthor][/authors]. ([date dateiso="20150000" specyear="2015"]2015[/date]). [source]Bridge collapses around the world: Causes and mechanisms[/source]. [confgrp]Paper presented at the [confname]IABSE-JSCE Joint Conference on Advances in Bridge Engineering-III[/confname], [city]Dhaka[/city], [country]Bangladesh[/country][/confgrp]. [/ref]

[ref id="r2" reftype=“journal"][authors role="nd"][pauthor][surname]Cook[/surname], [fname]W.[/fname][/pauthor], [pauthor][surname]Bar[/surname], [fname]P. J.[/fname][/pauthor], & [pauthor][surname]Halling[/surname], [fname]M. W[/fname][/pauthor][/authors]. ([date dateiso="20150000" specyear="2015"]2015[/date]). [arttitle]Bridge failure rate[/arttitle]. [source]Journal of Performance of Constructed Facilities[/source], [volid]29[/volid]([issueno]3[/issueno]), [pages]1-8[/pages]. doi:[url]https://doi.org/10.1061/(asce)cf.1943-5509.0000571[/url] [/ref]

[ref id="r3" reftype="book"][authors role="nd"][pauthor][surname]Cook[/surname], [fname]W.[/fname][/pauthor], [pauthor][surname]Barr[/surname], [fname]P. J.[/fname][/pauthor], & [pauthor][surname]Halling[/surname], [fname]M. W[/fname][/pauthor][/authors]. ([date dateiso="20140000" specyear="2014"]2014[/date]). [source]Segregation of bridge failure causes and consequences[/source]. [moreinfo]Paper presented at the TRB 2014[/moreinfo], [publoc]Washington, DC[/publoc]. [/ref]

[ref id="r4" reftype=“confproc"][authors role="nd"][pauthor][surname]Duntemann[/surname], [fname]J. F.[/fname][/pauthor], & [pauthor][surname]Subrizi[/surname], [fname]C. D[/fname][/pauthor][/authors]. ([date dateiso="20000000" specyear="2000"]2000[/date]). [source]Lessons learned from bridge construction failures[/source]. [confgrp]Paper presented at the [confname]Second Forensic Engineering Congress[/confname], [city]Puerto[/city][/confgrp]. [/ref]

[ref id="r5" reftype="webpage"][authors role="nd"][pauthor][surname]Guang[/surname], [fname]L[/fname][/pauthor][/authors]. ([date dateiso="20120000" specyear="2012"]2012[/date]). [url]https://www.cet.com.cn/dfpd/tsgd/594779.shtml[/url].   [/ref]

[ref id="r6" reftype=“journal"][authors role="nd"][pauthor][surname]Hager[/surname], [fname]W. H.[/fname][/pauthor], & [pauthor][surname]Unger[/surname], [fname]J[/fname][/pauthor][/authors]. ([date dateiso="20100000" specyear="2010"]2010[/date]). [arttitle]Bridge Pier Scour under Flood Waves[/arttitle]. [source]Journal of Hydraulic Engineering[/source], [volid]136[/volid]([issueno]10[/issueno]), [pages]842-847[/pages]. doi:[url]https://doi.org/10.1061/(ASCE)HY.1943-7900.0000281[/url][/ref]

[ref id="r7" reftype=“journal"][authors role="nd"][pauthor][surname]Harik[/surname], [fname]I. E.[/fname][/pauthor], [pauthor][surname]Shaaban[/surname], [fname]A. M.[/fname][/pauthor], [pauthor][surname]Gesund[/surname], [fname]H.[/fname][/pauthor], [pauthor][surname]Valli[/surname], [fname]G. Y. S.[/fname][/pauthor], & [pauthor][surname]Wang[/surname], [fname]S. T[/fname][/pauthor][/authors]. ([date dateiso="19900000" specyear="1990"]1990[/date]). [arttitle]United States bridge failures, 1951-1988[/arttitle]. [source]Journal of Performance of Constructed Facilities[/source], [volid]4[/volid]([issueno]4[/issueno]), 272-277. [/ref]

[ref id="r8" reftype=“journal"][authors role="nd"][pauthor][surname]Hong[/surname], [fname]J.-H.[/fname][/pauthor], [pauthor][surname]Chiew[/surname], [fname]Y.-M.[/fname][/pauthor], [pauthor][surname]Lu[/surname], [fname]J.-Y.[/fname][/pauthor], [pauthor][surname]Lai[/surname], [fname]J.-S.[/fname][/pauthor], & [pauthor][surname]Lin[/surname], [fname]Y.-B[/fname][/pauthor][/authors]. ([date dateiso="20120000" specyear="2012"]2012[/date]). [arttitle]Houfeng bridge failure in Taiwan[/arttitle]. [source]Journal of Hydraulic Engineering[/source], [volid]138[/volid]([issueno]2[/issueno]), [pages]186-198[/pages]. doi:[url]https://doi.org/10.1061/(ASCE)HY.1943-7900.0000430[/url][/ref]

[ref id="r9" reftype=“journal"][authors role="nd"][pauthor][surname]Hung[/surname], [fname]C.-C.[/fname][/pauthor], & [pauthor][surname]Yau[/surname], [fname]W.-G[/fname][/pauthor][/authors]. ([date dateiso="20140000" specyear="2014"]2014[/date]). [arttitle]Behavior of scoured bridge piers subjected to flood-induced loads[/arttitle]. [source]Engineering Structures[/source], [volid]80[/volid], [pages]241-250[/pages]. doi:[url]https://doi.org/10.1016/j.engstruct.2014.09.009[/url][/ref]

[ref id="r10" reftype=“journal"][authors role="nd"][pauthor][surname]Ji[/surname], [fname]B. H.[/fname][/pauthor], & [pauthor][surname]Fu[/surname], [fname]Z. Q[/fname][/pauthor][/authors]. ([date dateiso="20100000" specyear="2010"]2010[/date]). [arttitle]Analysis of Domestic Bridge Collapse Accidents In Recent Years[/arttitle]. [source]China Civil Engineering Journal[/source], [volid]43[/volid]([issueno]S[/issueno]), [pages]495-498[/pages]. doi:[url]https://doi.org/10.15951/j.tmgcxb.2010.s1.010[/url] ([moreinfo]in Chinese[/moreinfo])[/ref]

[ref id="r11" reftype=“journal"][authors role="nd"][pauthor][surname]Ko[/surname], [fname]Y.-Y.[/fname][/pauthor], [pauthor][surname]Chiou[/surname], [fname]J.-S.[/fname][/pauthor], [pauthor][surname]Tsai[/surname], [fname]Y.-C.[/fname][/pauthor], & [pauthor][surname]Chen[/surname], [fname]C.-H[/fname][/pauthor][/authors]. ([date dateiso="20140000" specyear="2014"]2014[/date]). [arttitle]Evaluation of Flood-Resistant Capacity of Scoured Bridges[/arttitle]. [source]Journal of Performance of Constructed Facilities[/source], [volid]28 [/volid]([issueno]1[/issueno]), [pages]61-75[/pages] doi: [url]https://doi.org/10.1061/(ASCE)CF.1943-5509.0000381[/url][/ref]

[ref id="r12" reftype=“journal"][authors role="nd"][pauthor][surname]Lin[/surname], [fname]C.[/fname][/pauthor], [pauthor][surname]Bennett[/surname], [fname]C.[/fname][/pauthor], [pauthor][surname]Han[/surname], [fname]J.[/fname][/pauthor], & [pauthor][surname]Parsons[/surname], [fname]R. L[/fname][/pauthor][/authors]. ([date dateiso="20120000" specyear="2012"]2012[/date]). [arttitle]Integrated analysis of the performance of pile-supported bridges under scoured conditions[/arttitle]. [source]Engineering Structures[/source], [volid]36[/volid], [pages]27-38[/pages]. doi: [url]https://doi.org/10.1016/j.engstruct.2011.11.015[/url][/ref]

[ref id="r13" reftype=“journal"][authors role="nd"][pauthor][surname]Madeleine[/surname] [fname]M. Flint[/fname][/pauthor], [pauthor][fname]Oliver[/fname] [surname]Fringe[/surname][/pauthor], [pauthor][surname]Sarah[/surname] [fname]L. Billington[/fname][/pauthor], [pauthor][fname]David[/fname] [surname]Freyberg[/surname][/pauthor], & [pauthor][surname]Diffenbaugh[/surname], [fname]N. S[/fname][/pauthor][/authors]. ([date dateiso="20170000" specyear="2017"]2017[/date]). [arttitle]Historical Analysis of Hydraulic Bridge Collapses in the Continental United States[/arttitle]. [source]Journal of Infrastructure Systems[/source], [volid]23[/volid]([issueno]3[/issueno]), [pages]1-16[/pages]. doi: [url]https://doi.org/10.1061/(ASCE)IS.1943-555X.0000354[/url][/ref]

[ref id="r14" reftype="webpage"][authors role="nd"][pauthor][surname]Mengqi[/surname], [fname]L[/fname][/pauthor][/authors]. ([date dateiso="20100000" specyear="2010"]2010[/date]). [url]http://news.sohu.com/20100819/n274335662.shtml[/url]. [/ref]

[ref id="r15" reftype="webpage"][authors role="nd"][pauthor][surname]Mingkang[/surname], [fname]L[/fname][/pauthor][/authors]. ([date dateiso="20100000" specyear="2010"]2010[/date]). [url]https://cd.qq.com/zt2010/chongzhou/[/url]. [/ref]

[ref id="r16" reftype=“journal"][authors role="nd"][pauthor][surname]Montalvo[/surname], [fname]C.[/fname][/pauthor], [pauthor][surname]Cook[/surname], [fname]W.[/fname][/pauthor], & [pauthor][surname]Keeney[/surname], [fname]T[/fname][/pauthor][/authors]. ([date dateiso="20200000" specyear="2020"]2020[/date]). [arttitle]Retrospective Analysis of Hydraulic Bridge Collapse[/arttitle]. [source]Journal of Performance of Constructed Facilities[/source], [volid]34[/volid]([issueno]1[/issueno]), [pages]1-8[/pages]. doi: [url]https://doi.org/10.1061/(ASCE)CF.1943-5509.0001378[/url][/ref]

[ref id="r17" reftype=“journal"][authors role="nd"][pauthor][surname]Moroni[/surname], [fname]M. O.[/fname][/pauthor], [pauthor][surname]Sarrazin[/surname], [fname]M.[/fname][/pauthor], [pauthor][surname]Venegas[/surname], [fname]B.[/fname][/pauthor], & [pauthor][surname]Villarroel[/surname], [fname]J[/fname][/pauthor][/authors]. ([date dateiso="20150000" specyear="2015"]2015[/date]). [arttitle]Seismic behavior of chilean bridges with seismic protection devices[/arttitle]. [source]Revista de la Construcción Journal of Construction[/source], [volid]1[/volid]([issueno]1[/issueno]). [/ref]

[ref id="r18" reftype=“journal"][authors role="nd"][pauthor][surname]Rodrigo[/surname], [fname]B. G[/fname][/pauthor]., [pauthor][surname]Olària[/surname], [fname]S. R. I[/fname][/pauthor]., [pauthor][surname]Fernández-Ordoñez[/surname], [fname]D[/fname][/pauthor]., & [pauthor][surname]Gómez[/surname], [fname]J. M. C.-S[/fname][/pauthor][/authors]. ([date dateiso="20150000" specyear="2015"]2015[/date]). [arttitle]Rehabilitation of Historic Masonry Bridges: Lessons Learned from a Medieval Bridge in Northeast Spain[/arttitle]. [source]Revista de la Construcción Journal of Construction[/source], [volid]14[/volid]([issueno]2[/issueno]), [pages]9-13[/pages]. [/ref]

[ref id="r19" reftype="webpage"][authors role="nd"][cauthor]Sina[/cauthor][/authors]. ([date dateiso="20000000" specyear="2000"]2000[/date]). [url]https://news.sina.com.cn/china/2000-08-28/121178.html[/url].   [/ref]

[ref id="r20" reftype=“journal"][authors role="nd"][pauthor][surname]Wardhana[/surname], [fname]K.[/fname][/pauthor], & [pauthor][surname]Hadipriono[/surname], [fname]F. C[/fname][/pauthor][/authors]. ([date dateiso="20030000" specyear="2003"]2003[/date]). [arttitle]Analysis of Recent Bridge Failures in the United States[/arttitle]. [source]Journal of Performance of Constructed Facilities[/source], [volid]17[/volid]([issueno]3[/issueno]), [pages]144-150 [/pages]doi: [url]https://doi.org/10.1061/(ASCE)0887-3828(2003)17:3(144)[/url][/ref]

[ref id="r21" reftype="webpage"][authors role="nd"][cauthor]Xinhuashe[/cauthor][/authors]. ([date dateiso="20100000" specyear="2010"]2010[/date]). [url]https://war.163.com/photoview/00AN0001/10072.html#p=6C3OLBSD00AN0001[/url]. [/ref]

[ref id="r22" reftype=“journal"][authors role="nd"][pauthor][surname]Xu[/surname], [fname]D[/fname][/pauthor][/authors]. ([date dateiso="20030000" specyear="2003"]2003[/date]). [arttitle]Accident Environmental Risk Assessment in Bridge Construction[/arttitle]. [source]China Safety Science Journal[/source], [volid]13[/volid]([issueno]8[/issueno]), [pages]46-49[/pages]. doi: [url]https://doi.org/10.16265/j.cnki.issn1003-3033.2003.08.012[/url] ([moreinfo]in Chinese[/moreinfo])[/ref]

[ref id="r23" reftype=“journal"][authors role="nd"][pauthor][surname]Xu[/surname], [fname]H. T.[/fname][/pauthor], [pauthor][surname]Guo[/surname], [fname]G. Z.[/fname][/pauthor], [pauthor][surname]Pu[/surname], [fname]H. L.[/fname][/pauthor], & [pauthor][surname]Yuan[/surname], [fname]M[/fname][/pauthor][/authors]. ([date dateiso="20070000" specyear="2007"]2007[/date]). [arttitle]Causes and lessons of bridge accidents in China in recent years[/arttitle]. [source]China Safety Science Journal[/source], [volid]17[/volid]([issueno]11[/issueno]), [pages]90-96[/pages]. doi: [url]https://doi.org/10.16265/j.cnki.issn1003-3033.2007.11.018 [/url]([moreinfo]in Chinese[/moreinfo])[/ref]

[ref id="r24" reftype="webpage"][authors role="nd"][cauthor]Zhaoting[/cauthor][/authors]. ([date dateiso="20100000" specyear="2010"]2010[/date]). [url]http://news.sohu.com/20100729/n273847112.shtml[/url]. [/ref]

[ref id="r25" reftype=“journal"][authors role="nd"][pauthor][surname]Zhou[/surname], [fname]X.[/fname][/pauthor], & [pauthor][surname]Zhang[/surname], [fname]X[/fname][/pauthor][/authors]. ([date dateiso="20190000" specyear="2019"]2019[/date]). [arttitle]Thoughts on the Development of Bridge Technology in China[/arttitle]. [source]Engineering[/source], [volid]5[/volid], [pages]1120-1130[/pages]. doi: [pubid idtype="doi"]https://doi.org/10.1016/j.eng.2019.10.001[/pubid][/ref]

[ref id="r26" reftype=“journal"][authors role="nd"][pauthor][surname]Zhuang[/surname], [fname]D.[/fname][/pauthor], [pauthor][surname]Xiao[/surname], [fname]R.[/fname][/pauthor], [pauthor][surname]Jia[/surname], [fname]L.[/fname][/pauthor], & [pauthor][surname]Sun[/surname], [fname]B[/fname][/pauthor][/authors]. ([date dateiso="20200000" specyear="2020"]2020[/date]). [arttitle]Failure analysis for overall stability against sliding and overturning of a girder bridge[/arttitle]. [source]Engineering Failure Analysis[/source], [volid]109[/volid], [pages]1-13[/pages]. doi: [pubid idtype="doi"]https://doi.org/10.1016/j.engfailanal.2019.104271[/pubid][/ref][/refs]

[/doc]